LOW LEVEL DESIGN (LLD)

SCCM to NSO CFS/RFS Service Provisioning Integration

MPLS L3VPN & GPON (Nokia/Tejas) Service Orchestration

Document Type: Low Level Design (LLD)

Systems: SCCM | NSO CFS | NSO RFS

# Document Control

| Document Title | SCCM–NSO Integration: Service Order Provisioning & Validation (LLD) |
| --- | --- |
| Document Type | Low Level Design (LLD) |
| Services Covered | MPLS L3VPN, GPON (Nokia/Tejas) |
| Systems Involved | SCCM, NSO CFS, NSO RFS |
| Interface Protocol | RESTCONF |

# Table of Contents

# 1. Purpose

This document provides the Low Level Design (LLD) for service order provisioning between SCCM (Service Configuration & Change Management) and Cisco NSO. It details the internal SCCM API sequence, the RESTCONF payload structure exchanged with NSO CFS, and the field-level validation steps performed in NSO CFS and NSO RFS to confirm successful service orchestration.

# 2. Scope

This LLD covers the following in detail:

- SCCM's 3-step internal API workflow for service order creation (Transaction ID lifecycle)

- RESTCONF payload generation and transfer from SCCM to NSO CFS

- Field-level configuration data for MPLS L3VPN and GPON (Nokia/Tejas) service types

- Post-orchestration validation steps and expected values across NSO CFS and NSO RFS

# 3. Architecture Overview

The end-to-end flow moves from SCCM's internal order-processing APIs, through RESTCONF-based service orchestration in NSO CFS, down to device-level configuration in NSO RFS:

- SCCM (Step 1 → Step 2 → Step 3) generates and threads a single Transaction ID

- SCCM builds two RESTCONF payloads: MPLS L3VPN and GPON (Nokia/Tejas)

- NSO CFS receives payloads and creates corresponding service instances

- NSO CFS pushes device-level configuration to NSO RFS-managed network devices

- SCCM order status is updated to Committed once orchestration succeeds

# 4. SCCM Service Order Creation Workflow

Three sequential internal API calls create and process the service order. A single Transaction ID, generated in Step 1, threads through all three calls.

## 4.1 Step 1 – Service Order Initialization

| Purpose | Creates a new service deployment request and generates the Transaction ID used by Steps 2 and 3 |
| --- | --- |
| Input | Service Order ID, Customer ID, Service Type, Location, VLAN, other service attributes |
| Output | Unique Transaction ID |

Outcome:

- Service order request is successfully initialized

- Transaction record is created in SCCM

- Transaction ID is returned to the calling application

- Transaction ID is reused in Step 2 (data submission) and Step 3 (provisioning)

## 4.2 Step 2 – Service Order Data Submission

| Purpose | Submits full service-specific configuration against the Step 1 Transaction ID |
| --- | --- |
| Input | Transaction ID; endpoint device count; service-specific ID; upstream/downstream bandwidth profiles; ONT details (model, serial number, PON port, tagging information); OLT hostname; PE hostname and PE port; other service-specific parameters |
| Output | Service configuration validated and linked to the Transaction ID for further processing |

Outcome:

- Service order details submitted against the Step 1 Transaction ID

- Endpoint device and service configuration information processed by SCCM

- ONT, OLT, bandwidth profile, and PE connectivity details validated

- Transaction ID remains active for use in Step 3

## 4.3 Step 3 – Service Provisioning

| Purpose | Initiates the service provisioning process and triggers downstream configuration activities using the Transaction ID |
| --- | --- |
| Input | Transaction ID generated in Step 1; Action value indicating the provisioning operation; service order information submitted in Step 2 |
| Output | Service provisioning status; unique service identifier generated for the provisioned service |

Outcome:

- SCCM retrieves service order details associated with the Transaction ID

- Service provisioning workflow is initiated and required configuration processing triggered

- A unique service identifier is generated for the provisioned service

- SCCM returns a successful response indicating the provisioning request has been accepted

Generated Service Identifier: CKT-L3VPN-GI-BB-SCCM-Tik100566

# 5. RESTCONF API Integration with NSO CFS

After successful completion of Steps 1–3, SCCM consolidates the collected information — service order details, customer information, VLAN configuration, bandwidth profiles, OLT/ONT details, PE device details, VRF configuration, routing information, and QoS policies — and generates two separate RESTCONF payloads sent to NSO CFS.

| Payload | Target Configuration | Consumed By |
| --- | --- | --- |
| MPLS L3VPN RESTCONF Payload | Router configuration: Circuit ID, VPN ID, VRF definitions, interface config, VLAN info, QoS policies, routing parameters, PE device details | NSO CFS |
| GPON RESTCONF Payload (Nokia/Tejas) | OLT/ONT configuration: OLT info, ONT details/serial number, VLAN mapping, bandwidth profiles, subscriber information | NSO CFS |

## 5.1 MPLS L3VPN RESTCONF Payload

| Purpose | Transfers MPLS L3VPN service configuration from SCCM to NSO CFS for orchestration and configuration deployment |
| --- | --- |
| Input Elements | Transaction ID; Service ID; Circuit ID; VPN ID; Sub Service ID; Customer VLAN Information; PE Device Information; Interface Configuration; VRF Configuration; QoS Configuration; Routing Parameters; WAN IP Address Information |
| Output | MPLS L3VPN RESTCONF payload generated and forwarded to NSO CFS |

Outcome:

- Payload generated using service, interface, VLAN, VRF, QoS, and routing configuration details

- RESTCONF API triggered towards NSO CFS; payload received and processed

- MPLS L3VPN orchestration workflow initiated; configuration staged for deployment on target PE devices

## 5.2 GPON RESTCONF Payload (Nokia/Tejas)

| Purpose | Transfers GPON access service configuration from SCCM to NSO CFS for orchestration on Nokia/Tejas OLT infrastructure |
| --- | --- |
| Input Elements | Transaction ID; Service ID; Circuit ID; OLT Information; ONT Information; ONT Serial Number; ONT Card and Port Details; Customer VLAN Information; Network VLAN Information; Upstream Bandwidth Profile; Downstream Bandwidth Profile; Subscriber Information; Service-Specific Parameters |
| Output | GPON RESTCONF payload generated and forwarded to NSO CFS |

Outcome:

- Payload contains OLT, ONT, VLAN, bandwidth profile, and subscriber-specific configuration details

- RESTCONF API triggered towards NSO CFS; payload received and processed

- GPON orchestration workflow initiated on target Nokia/Tejas OLT infrastructure

# 6. Service Order Status Verification in SCCM

After Steps 1–3 and RESTCONF-based orchestration in NSO CFS complete, the service configurations are applied on the target network infrastructure. NSO updates the provisioning status back to SCCM, and SCCM updates the service order status to Committed, confirming the workflow has been completed successfully.

# 7. MPLS L3VPN Service Validation in NSO CFS

This section verifies that the MPLS L3VPN configuration received from SCCM via RESTCONF has been successfully processed and instantiated in NSO CFS.

## 7.1 Service Instance Validation

Navigation: NSO CFS Portal → MPLS L3VPN Service Manager → search by Service ID → open Circuit ID.

| Validation Point | Expected Value |
| --- | --- |
| Service ID | Tik100566 |
| Circuit ID | CKT-L3VPN-GI-BB-SCCM-Tik100566 |
| Attributes to cross-check | Circuit ID, Sub Service ID, VPN ID, Site ID, Topology Name, Routing Protocol, PE Type |

Outcome:

- MPLS L3VPN service instance available and searchable via Service ID in NSO CFS Service Manager

- Service attributes match values received via the SCCM RESTCONF payload

- Service instance available for subsequent synchronization, deployment, and lifecycle management

## 7.2 Configuration Editor — Circuit ID Drill-down

Opening the Circuit ID within the service instance displays the detailed configuration received from SCCM via RESTCONF.

- Device configuration details are associated with the service instance

- VRF configuration is created and available for verification

- Static routing configuration is populated in the service instance

- QoS configuration is applied and available for validation

## 7.3 Interface Configuration Verification

Navigation: Select the required interface row (e.g., BDI3301 or Port-channel21) within the service instance.

| Field | Example Value |
| --- | --- |
| Interface | BDI3301 / Port-channel21 |
| Connected Device | NOCTESTGAC001 |
| Transaction ID | 5776 |
| Customer VLAN ID | 3301 |
| Attributes checked | Interface ID, Device Mode, IP Type, VRF Name, WAN IP Address, Service Policy, Service Instance Number |

## 7.4 Device Configuration Verification

Navigation: Select the required device row within the service instance.

| Field | Example Value |
| --- | --- |
| Device | NOCTESTGAC001 |
| Transaction ID | 5776 |
| VRF Name | GI_BB_SERVICES_NSO |
| Attributes checked | BGP ID, Provider AS Number, IP Type |

## 7.5 QoS Configuration Verification

Navigation: Service instance → qos-config section → select device row.

| Field | Example Value |
| --- | --- |
| Device | NOCTESTGAC001 |
| QoS Name | COS_BRONZE |
| Transaction ID | 5776 |
| Class Map Name | GI_VLAN_3301 |
| Policy Map Name | Ingress-policy-GI |
| QoS Group Value | 1 |
| Class Map VLAN | 3301 |

## 7.6 VRF Configuration Verification

Navigation: Service instance → vrf-definitions section → select device row.

| Field | Example Value |
| --- | --- |
| Device | NOCTESTGAC001 |
| VRF Name | GI_BB_SERVICES_NSO |
| Transaction ID | 5776 |
| Address Family | both |
| Route Distinguisher | 172.17.128.1:93 |
| Import Route Target | 141688:32 |
| Export Route Target | 141688:31 |

# 8. GPON Service Instance Validation in NSO CFS

Navigation: NSO CFS → Nokia Service Manager → search by Service ID → open service configuration.

| Field | Value |
| --- | --- |
| GPON Service ID | Tik100566-GPON-180626120031 |
| Circuit ID | CKT-L3VPN-GI-BB-SCCM-Tik100566 |
| Nokia OLT Device | nokia-ams-pkga |
| Template Name | ORG_SERV_U_D_CVLAN |
| Service Status | ACTIVE |
| ONT Serial Number | TJNW95D2CD14 |
| Upstream Bandwidth Profile | UP_TNET_GI_50M |
| Downstream Bandwidth Profile | DN_TNET_GI_50M |
| Customer VLAN | 2537 |
| Network VLAN | GI-Internet_S3301 |

Outcome: Subscriber IDs, VLAN mapping, and bandwidth profiles all reflect the values received through the SCCM RESTCONF payload.

# 9. Device Verification in NSO RFS

After successful orchestration from NSO CFS, verify the target device in the NSO RFS Device Manager.

Navigation: NSO RFS → Device Manager → search device (NOCTESTGAC001) → open device configuration.

| Field | Value |
| --- | --- |
| Device | NOCTESTGAC001 |
| IP Address | 172.17.128.1 |
| Port | 22 |
| Device Type | Cisco IOS CLI |
| Verification | Service List entries reflect associated MPLS L3VPN service configurations |

# 10. End-to-End Flow Summary

- SCCM: Step 1 → Step 2 → Step 3 (Transaction ID created and carried through)

- SCCM generates MPLS L3VPN and GPON RESTCONF payloads

- NSO CFS receives payloads and creates service instances (MPLS L3VPN Service Manager / Nokia Service Manager)

- SCCM order status updated to Committed

- Validation performed in NSO CFS (service, interface, device, QoS, VRF) and NSO RFS (device-level configuration)