ACCEPTANCE TEST PROCEDURE

(ATP)

GI-BB-NOKIA-SINGLE-ASR920-ROUTE

&

GI-BB-TEJAS-SINGLE-ASR907-ROUTE

GPON Broadband Service Provisioning — SCCM to NSO CFS/RFS Orchestration

(Nokia OLT and Tejas EMS Device Variants)

Service Order IDs: S109702026-09050204-022073 (Nokia)  |  S109702026-09040205-022169 (Tejas)

| Parameter | Value |
| --- | --- |
| Document Title | Acceptance Test Procedure – GI-BB-NOKIA-SINGLE-ASR920-ROUTE & GI-BB-TEJAS-SINGLE-ASR907-ROUTE |
| Service Order ID (Nokia) | S109702026-09050204-022073 |
| Service Order ID (Tejas) | S109702026-09040205-022169 |
| Customer ID | GI00000893 |
| Document Version | 1.0 |
| Document Status | Draft |
| Date | 17-Jul-2026 |
| Prepared By |  |
| Reviewed By |  |
| Approved By |  |

# Table of Contents

# 1. Introduction

## 1.1 Purpose

This document defines the Acceptance Test Procedure (ATP) for two GI-BB broadband GPON service variants: GI-BB-NOKIA-SINGLE-ASR920-ROUTE (Nokia OLT access infrastructure) and GI-BB-TEJAS-SINGLE-ASR907-ROUTE (Tejas EMS access infrastructure). It validates end-to-end service provisioning through the Service Configuration and Circuit Management (SCCM) application, orchestration via NSO CFS and NSO RFS, and deployment verification on the respective target access network infrastructure.

## 1.2 Scope

This ATP is organized into two parts, one for each device variant:

- Part A — GI-BB-NOKIA-SINGLE-ASR920-ROUTE (Service Order S109702026-09050204-022073): SCCM Step 1/2/3 provisioning, MPLS L3VPN and GPON RESTCONF payload transmission to NSO CFS, ONT/VLAN verification on the Nokia OLT, and subscriber bandwidth validation.

- Part B — GI-BB-TEJAS-SINGLE-ASR907-ROUTE (Service Order S109702026-09040205-022169): SCCM Step 1/2/3 provisioning, MPLS L3VPN RESTCONF payload transmission to NSO CFS, MPLS L3VPN service verification in NSO CFS and NSO RFS, and ONT creation/status verification in Tejas EMS.

## 1.3 Reference Details

### Nokia Device Variant

| Parameter | Value |
| --- | --- |
| Service Name | GI-BB-NOKIA-SINGLE-ASR920-ROUTE |
| Service Order ID | S109702026-09050204-022073 |
| Customer | GI00000893 |
| Reference ID | 9.5.2.4 |
| Order Type | Create |
| Topology | Spoke |
| Transaction ID | 180 |
| Circuit ID | AC000E8822-01-04-000518-TL0001 |
| Test Environment User | bss_test_user |

### Tejas Device Variant

| Parameter | Value |
| --- | --- |
| Service Name | GI-BB-TEJAS-SINGLE-ASR907-ROUTE |
| Service Order ID | S109702026-09040205-022169 |
| Customer | GI00000893 |
| Reference ID | 9.4.2.5 |
| Order Type | Create |
| Topology | Spoke |
| Transaction ID | 225 |
| Circuit ID | AC000E8930-01-04-000518-TL0005 |
| Test Environment User | bss_test_user |

## 1.4 Test Case Summary

| Device | Test Case | Title | Objective |
| --- | --- | --- | --- |
| Nokia | TC-01 | Step 1 – Service Order Initialization | Validate SCCM service order creation and Transaction ID generation |
| Nokia | TC-02 | Step 2 – Service Order Data Submission | Validate service/device parameters submitted against the Transaction ID |
| Nokia | TC-03 | Step 3 – Service Provisioning | Validate service provisioning initiation and Unique ID generation |
| Nokia | TC-04 | MPLS L3VPN RESTCONF Payload to NSO CFS | Validate MPLS L3VPN payload generation and transmission to NSO CFS |
| Nokia | TC-05 | GPON RESTCONF Payload to NSO CFS | Validate GPON payload generation and transmission to NSO CFS |
| Nokia | TC-06 | ONT Status, ONT Details & VLAN Verification | Validate ONT provisioning and VLAN mapping on the Nokia OLT |
| Nokia | TC-07 | Internet Bandwidth Validation | Validate end-to-end throughput against the provisioned bandwidth profile |
| Tejas | TC-08 | Service Order Creation and Step 1 API Execution | Validate SCCM service order creation and Transaction ID generation |
| Tejas | TC-09 | Step 2 API Execution and Service Data Submission | Validate service/ONT/OLT/PE parameters submitted against the Transaction ID |
| Tejas | TC-10 | Step 3 API Execution and Service Provisioning | Validate service provisioning initiation and Unique ID generation |
| Tejas | TC-11 | MPLS L3VPN RESTCONF Payload to NSO CFS | Validate MPLS L3VPN payload generation and transmission to NSO CFS |
| Tejas | TC-12 | MPLS L3VPN Service Creation in NSO CFS | Validate MPLS L3VPN service instance creation in NSO CFS |
| Tejas | TC-13 | MPLS L3VPN Service Creation in NSO RFS | Validate MPLS L3VPN service instance creation in NSO RFS |
| Tejas | TC-14 | ONT Creation and Status in Tejas EMS | Validate ONT provisioning and operational status in Tejas EMS |

# 2. Detailed Test Cases

## Part A — GI-BB-NOKIA-SINGLE-ASR920-ROUTE (Nokia Device Variant)

## TC-01: Step 1 – Service Order Initialization

Service Order Reference: S109702026-09050204-022073

Description

Verify that the service order is successfully created in SCCM and the corresponding Step 1 API transaction details are available in the API Log. Validate that the generated Transaction ID, request payload, and API response are correctly recorded.

Procedure

- Login to SCCM application.

- Navigate to Service Orders.

- Search for the Service Order ID: S109702026-09050204-022073.

- Verify that the service order is displayed in the Service Orders list.

- Navigate to API Logs.

- Open the API log entry associated with the Service Order.

- Verify the Step 1 API request details.

- Verify the generated Transaction ID and API response details.

Validation Points

### Service Order Details

| Parameter | Value |
| --- | --- |
| Service Order ID | S109702026-09050204-022073 |
| Ref ID | 9.5.2.4 |
| Customer | GI00000893 |
| Order Type | Create |
| Service Name | GI-BB-NOKIA-SINGLE-ASR920-ROUTE |
| Topology | Spoke |
| Status | Committed |

### Step 1 API Log

| Parameter | Value |
| --- | --- |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step1 |
| Request Method | POST |
| Transaction ID | 180 |
| User | bss_test_user |
| Message | Service Invoke Create Step1 Success |

### Request Payload Must Contain

- Service Order ID

- Customer ID

- Sub Service ID

- Endpoint Device Count

- Topology

- Location ID

- Service VLAN ID

### Response Must Contain

- Status Code = 2001

- Sub Status Code = 2001

- Transaction ID = 180

- Message: Service Invoke Create Step1 Success

Expected Result - Step 1 API request and response details are recorded in the API Log. - A unique Transaction ID is generated and returned by the API. - The Transaction ID is available for subsequent service provisioning steps.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Step 1 – SCCM Service Order / API Log Screenshot (Nokia)

## TC-02: Step 2 – Service Order Data Submission

Service Order Reference: S109702026-09050204-022073

Description

Verify that the Step 2 Service Order Data Submission API is successfully executed in SCCM and that the service provisioning details are processed against the Transaction ID generated in Step 1.

Procedure

- Login to SCCM application.

- Navigate to API Logs.

- Search for the Transaction ID: 180.

- Open the Step 2 API log entry.

- Verify the request payload submitted to the Step 2 API.

- Verify the response received from SCCM.

- Validate that the service-specific and device-specific parameters are associated with the Transaction ID.

Validation Points

### Step 2 API Log

| Parameter | Value |
| --- | --- |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step2 |
| Request Method | POST |
| Transaction ID | 180 |
| User | bss_test_user |
| API Duration | 1.623 sec |

### Request Payload Must Contain

- Endpoint Device Count = 1

- Circuit ID

- Transaction ID = 180

- Service Specific ID = S109702026-09050204-022073

- Upstream Bandwidth Profile = UP_TNET_GI_50M

- Downstream Bandwidth Profile = DN_TNET_GI_50M

- ONT Description

- ONT Tag = untagged

- ONT PON Port

- ONT Serial Number

- ONT Model

- OLT Host Name = nokia-ams-pkga

- PE Host Name = NOCTESTGAC001

- PE Port = 0/0/0

### Response Must Contain

| Parameter | Value |
| --- | --- |
| Status Code | 2002 |
| Sub Status Code | 2002 |
| Message | Service Invoke Create Step2 Success |
| Transaction ID | 180 |
| Circuit ID | AC000E8822-01-04-000518-TL0001 |

Expected Result - Step 2 API request is processed successfully. - Service-specific configuration details are associated with Transaction ID 180. - Endpoint device, ONT, OLT, bandwidth profile, and PE device information are accepted by SCCM. - Circuit ID AC000E8822-01-04-000518-TL0001 is generated and returned in the API response. - The transaction is updated and becomes available for Step 3 service provisioning.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Step 2 – SCCM API Log Screenshot (Nokia)

## TC-03: Step 3 – Service Provisioning

Service Order Reference: S109702026-09050204-022073

Description

Verify that the Step 3 Service Provisioning API is successfully executed in SCCM and that the service provisioning process is initiated using the Transaction ID generated in Step 1 and the service information submitted in Step 2.

Procedure

- Login to SCCM application.

- Navigate to API Logs.

- Search for the Transaction ID: 180.

- Open the Step 3 API log entry.

- Verify the request payload submitted to the Step 3 API.

- Verify the response received from SCCM.

- Validate that the service provisioning request is processed successfully.

Validation Points

### Step 3 API Log

| Parameter | Value |
| --- | --- |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step3 |
| Request Method | POST |
| Transaction ID | 180 |
| User | bss_test_user |
| API Duration | 0.023 sec |

### Request Payload Must Contain

- Transaction ID = 180

- Action = 1

- ReqDtlID = 3150

### Response Must Contain

| Parameter | Value |
| --- | --- |
| Status Code | 2000 |
| Sub Status Code | 2000 |
| Message | Service Invoke Create Step3 Success |
| Transaction ID | 180 |
| Unique ID | AC000E8822-01-04-000518-TL0001 |
| ReqDtlID | 3150 |

Expected Result - Step 3 API request is processed successfully. - Service provisioning workflow is initiated in SCCM. - Transaction ID 180 is successfully processed. - Unique Service Identifier AC000E8822-01-04-000518-TL0001 is generated and returned. - SCCM completes the service provisioning step and prepares the service order for downstream RESTCONF integration with NSO CFS.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Step 3 – SCCM API Log Screenshot (Nokia)

## TC-04: MPLS L3VPN RESTCONF Payload Generation and Transmission to NSO CFS

Service Order Reference: S109702026-09050204-022073

Description

Verify that after successful execution of SCCM Service Provisioning APIs (Step 1, Step 2, and Step 3), the MPLS L3VPN RESTCONF payload is generated and transmitted to NSO CFS. Validate that the payload contains the service, interface, VRF, QoS, and routing configuration details derived from the service order inputs.

Procedure

- Login to SCCM.

- Navigate to API Logs.

- Search for Transaction ID 180.

- Open the RESTCONF API log entry.

- Verify the RESTCONF API URL and request method.

- Review the generated MPLS L3VPN payload.

- Validate that service, interface, VRF, QoS, and routing details are present in the payload.

- Verify that the payload is forwarded towards the NSO CFS RESTCONF endpoint.

Validation Points

### RESTCONF API Details

| Parameter | Value |
| --- | --- |
| Transaction ID | 180 |
| Request Method | PATCH |
| API URL | http://172.28.2.1:8080/restconf/data/.../mpls-l3vpn-cfs |
| Created By | bss_test_user |

### Service Details in Payload

| Parameter | Value |
| --- | --- |
| Service ID | S109702026-09050204-022073 |
| Customer Name | GI00000893 |
| Circuit ID | AC000E8822-01-04-000518-TL0001 |
| VPN ID | 20447 |
| Site ID | RFS-18 |
| Topology Name | topology1 |
| Routing Protocol | static |
| PE Type | single |
| Sub Service ID | 9.5.2.4 |

### Interface Configuration Must Contain

- Connected Device = NOCTESTGAC001

- Interface ID = Port-channel21

- Transaction ID = 180

- Service Policy Name = Ingress-policy-GI

- Customer VLAN = 3301

- Service Instance Number = 3301

- VRF Name = GI_BB_SERVICES_NSO

- WAN IP Address = 10.11.185.209

### VRF Configuration Must Contain

- Device = NOCTESTGAC001

- VRF Name = GI_BB_SERVICES_NSO

- Transaction ID = 180

- Address Family = both

- Route Distinguisher = 172.17.128.1:113

- Import Route Target = 141688:32

- Export Route Target = 141688:31

### QoS Configuration Must Contain

- Device = NOCTESTGAC001

- QoS Name = COS_BRONZE

- Transaction ID = 180

- Class Map Name = GI_VLAN_3301

- Policy Map Name = Ingress-policy-GI

- QoS Group Value = 1

- Class Map VLAN = 3301

### Static Routing Configuration Must Contain

- Device = NOCTESTGAC001

- Transaction ID = 180

- BGP ID = 141688

- Provider AS Number = 141688

- IP Type = IPv4

- VRF Name = GI_BB_SERVICES_NSO

- Import Path Required = Yes

Expected Result - MPLS L3VPN RESTCONF payload is successfully generated by SCCM. - The payload contains service, interface, VRF, QoS, and static routing configuration details. - The RESTCONF API is invoked using the PATCH method. - The payload is successfully transmitted to NSO CFS for MPLS L3VPN service orchestration. - The configuration data present in the payload is available for further processing within NSO CFS.

[ SCREENSHOT / IMAGE PLACEHOLDER ] MPLS L3VPN RESTCONF Payload Screenshot (Nokia)

## TC-05: GPON RESTCONF Payload Generation and Transmission to NSO CFS

Service Order Reference: S109702026-09050204-022073

Description

Verify that after successful execution of SCCM Service Provisioning APIs (Step 1, Step 2, and Step 3), the GPON RESTCONF payload is generated and transmitted to NSO CFS. Validate that the payload contains the OLT, ONT, VLAN, bandwidth profile, and subscriber configuration details derived from the service order inputs.

Procedure

- Login to SCCM.

- Navigate to API Logs.

- Search for Transaction ID 180.

- Open the GPON RESTCONF API log entry.

- Verify the RESTCONF API URL and request method.

- Review the generated GPON payload.

- Validate that OLT, ONT, VLAN, subscriber, and bandwidth profile details are present in the payload.

- Verify that the payload is forwarded towards the NSO CFS RESTCONF endpoint.

Validation Points

### RESTCONF API Details

| Parameter | Value |
| --- | --- |
| Transaction ID | 180 |
| Request Method | PATCH |
| API URL | http://172.28.2.1:8080/restconf/data/.../nokia-apc-cfs |
| User | bss_test_user |

### Service Details in Payload

| Parameter | Value |
| --- | --- |
| Service ID | S109702026-09050204-022073-GPON-030726202259 |
| Circuit ID | AC000E8822-01-04-000518-TL0001 |
| Device | nokia-ams-pkga |
| Template Name | TEMPLATE-GI-BB-ROUTE |
| Template Version | 1 |
| Instance Label | 2 |
| Transaction ID | 180 |
| Customer Name | GI00000893 |

### ONT Configuration Must Contain

- ONT Serial Number = ALCFLBE50893

- ONT Card ID = 14

- Number of Data Ports = 1

- Ethernet Port ID = 1

### Bandwidth Profile Configuration Must Contain

- Upstream Bandwidth Profile = UP_TNET_GI_50M

- Downstream Bandwidth Profile = DN_TNET_GI_50M

### VLAN Configuration Must Contain

- Customer VLAN ID = 2538

- Network VLAN Name = GI-Internet_S3301

### Subscriber Information Must Contain

- ONT Subscriber ID 1 = GI-1-Begumpet_000518ONTC000076

- ONT Subscriber ID 2 = S109702026-09050204-022073-GIBB_2538-50Mbps

Expected Result - GPON RESTCONF payload is successfully generated by SCCM. - The payload contains OLT, ONT, VLAN, subscriber, and bandwidth profile configuration details. - The RESTCONF API is invoked using the PATCH method. - The payload is successfully transmitted to NSO CFS for GPON service orchestration. - The configuration data present in the payload is available for further processing within NSO CFS.

[ SCREENSHOT / IMAGE PLACEHOLDER ] GPON RESTCONF Payload Screenshot (Nokia)

## TC-06: ONT Status, ONT Details, and VLAN Configuration Verification on Nokia OLT

Service Order Reference: S109702026-09050204-022073

Description

Verify that the GPON service configuration provisioned through SCCM and orchestrated via NSO is successfully deployed on the target Nokia OLT. Validate the ONT operational status, ONT service details, Customer VLAN (CVLAN), and Service VLAN (SVLAN) configuration on the device.

Procedure

- Login to the Nokia OLT CLI.

- Execute the ONT status verification command: show equipment ont status pon ont 1/1/1/1/2

- Verify the ONT operational status and serial number.

- Execute the ONT detailed verification command: show equipment ont status pon ont 1/1/1/1/2 detail

- Verify the ONT description, subscriber details, and signal information.

- Execute the bridge port configuration verification command: info configure bridge port 1/1/1/1/2/14/1

- Verify the Customer VLAN and Service VLAN mappings.

Validation Points

### ONT Status Verification

| Parameter | Value |
| --- | --- |
| PON | 1/1/1/1 |
| ONT | 1/1/1/1/2 |
| Serial Number | ALCL:FBE50893 |
| Admin Status | Up |
| Operational Status | Up |
| OLT RX Signal Level | -23.6 dBm |
| ONT–OLT Distance | 0.1 |
| Description 1 | GI--1-Begumpet_000518ONTC000076 |
| Description 2 | S109702026-09050204-022073 |

### ONT Detailed Verification

| Parameter | Value |
| --- | --- |
| PON | 1/1/1/1 |
| ONT | 1/1/1/1/2 |
| Serial Number | ALCL:FBE50893 |
| Admin Status | Up |
| Operational Status | Up |
| OLT RX Signal Level | -24.5 dBm |
| ONT–OLT Distance | 0.1 |
| Description 1 | GI--1-Begumpet_000518ONTC000076 |
| Description 2 | S109702026-09050204-022073-GIBB_2538-50Mbps |

### VLAN Configuration Verification

| Parameter | Value |
| --- | --- |
| Bridge Port | 1/1/1/1/2/14/1 |
| Customer VLAN (CVLAN) | 2538 |
| VLAN Tagging | Single Tagged |
| Service VLAN (SVLAN) | 3301 |
| VLAN Scope | Local |

Expected Result - ONT is successfully provisioned on the Nokia OLT. - ONT Administrative Status is displayed as Up. - ONT Operational Status is displayed as Up. - ONT Serial Number matches the value provisioned through SCCM and NSO. - ONT descriptions are populated with the subscriber and service identifiers. - Customer VLAN (2538) is configured on the ONT service port. - Service VLAN (3301) is mapped to the customer VLAN. - The OLT configuration confirms that the GPON service configuration received from SCCM through NSO has been successfully deployed on the target device.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Nokia OLT CLI – ONT Status / ONT Details / SVLAN & CVLAN Screenshots

## TC-07: Internet Bandwidth Validation After Service Provisioning

Service Order Reference: S109702026-09050204-022073

Description

Verify that internet connectivity is successfully established after service provisioning and that the provisioned bandwidth profile is reflected during throughput testing. Execute a speed test from the subscriber endpoint and validate the download and upload speeds.

Procedure

- Connect a client device through the provisioned GPON service.

- Ensure internet connectivity is available.

- Open a web browser and navigate to the Speedtest application.

- Execute the internet speed test.

- Observe the download speed, upload speed, and latency values.

- Compare the results with the provisioned bandwidth profile.

Validation Points

### Speed Test Results

| Parameter | Value |
| --- | --- |
| Service Provider | Aeronet Online Services Pvt Ltd |
| Server Location | Hyderabad |
| Download Speed | 48.90 Mbps |
| Upload Speed | 48.73 Mbps |
| Ping | 6 ms |
| Download Latency | 146 ms |
| Upload Latency | 29 ms |

### Additional Checks

- Internet connectivity is available through the provisioned service.

- Download traffic is successfully passing through the service.

- Upload traffic is successfully passing through the service.

- The observed bandwidth is aligned with the configured GPON bandwidth profile.

- Service traffic is routed successfully through the provisioned network path.

Expected Result - Internet connectivity is established successfully after service provisioning. - The subscriber is able to access external internet resources. - Download and upload traffic flow successfully through the provisioned GPON service. - The measured bandwidth reflects the configured service profile. - End-to-end service provisioning is validated from SCCM through NSO orchestration and network deployment.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Speed Test Result Screenshot (Nokia)

## Part B — GI-BB-TEJAS-SINGLE-ASR907-ROUTE (Tejas Device Variant)

## TC-08: Verify Service Order Creation and Step 1 API Execution

Service Order Reference: S109702026-09040205-022169

Description

Verify that the service order S109702026-09040205-022169 is successfully created in SCCM and that the Step 1 API is executed successfully with a Transaction ID generated for subsequent provisioning activities.

Procedure

- Login to SCCM.

- Navigate to Service Orders.

- Search for Service Order ID: S109702026-09040205-022169.

- Verify the service order entry is displayed.

- Verify the service status is initially displayed as Step 1.

- Navigate to API Logs.

- Open the Step 1 API log associated with Transaction ID 225.

- Verify the request payload and response details.

Validation Points

### Service Order Verification

| Parameter | Value |
| --- | --- |
| Service Order ID | S109702026-09040205-022169 |
| Ref ID | 9.4.2.5 |
| Customer ID | GI00000893 |
| Order Type | Create |
| Service Name | GI-BB-TEJAS-SINGLE-ASR907-ROUTE |
| Topology | Spoke |
| Status | Step 1 |

### Step 1 API Verification

| Parameter | Value |
| --- | --- |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step1 |
| Request Method | POST |
| Transaction ID | 225 |
| User | bss_test_user |
| Message | Service Invoke Create Step1 Success |

### Request Payload Must Contain

- Service Order ID = S109702026-09040205-022169

- Customer ID = GI00000893

- Sub Service ID = 9.4.2.5

- Endpoint Device Count = 1

- QoS = false

- Topology = 5

- Location ID = RANGAREDDY

- Service VLAN ID = 9

### Response Verification

| Parameter | Value |
| --- | --- |
| Status Code | 2001 |
| Sub Status Code | 2001 |
| Message | Service Invoke Create Step1 Success |
| Transaction ID | 225 |
| Service ID | S109702026-09040205-022169 |

Expected Result - Service order S109702026-09040205-022169 is successfully created in SCCM. - Step 1 API is executed successfully. - Transaction ID 225 is generated and returned in the API response. - The generated Transaction ID is available for Step 2 and Step 3 provisioning activities. - Service order status is displayed as Step 1 during the provisioning workflow.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Service Order Creation and Step 1 API Validation for GI-BB-TEJAS-SINGLE-ASR907-ROUTE

## TC-09: Verify Step 2 API Execution and Service Data Submission

Service Order Reference: S109702026-09040205-022169

Description

Verify that the Step 2 Service Order Data Submission API is successfully executed for Service Order S109702026-09040205-022169 and that the service-specific, ONT, OLT, bandwidth profile, and PE device details are successfully processed against the Transaction ID generated in Step 1.

Procedure

- Login to SCCM.

- Navigate to API Logs.

- Search for Transaction ID 225.

- Open the Step 2 API log entry.

- Verify the request payload details.

- Verify the response received from SCCM.

- Validate that the service and device-related parameters are associated with the transaction.

Validation Points

### API Details Verification

| Parameter | Value |
| --- | --- |
| Transaction ID | 225 |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step2 |
| Request Method | POST |
| User | bss_test_user |
| Message | Service Invoke Create Step2 Success |

### Service Information Verification

| Parameter | Value |
| --- | --- |
| Service Specific ID | S109702026-09040205-022169 |
| Transaction ID | 225 |
| Endpoint Device Count | 1 |
| Circuit ID | AC000E8930-01-04-000518-TL0005 |

### ONT Configuration Verification

| Parameter | Value |
| --- | --- |
| ONT Serial Number | TJNW39501D3F |
| ONT Model | TJ210N-11G |
| ONT Tag | tagged |
| ONT PON Port | NOCTESTOLTN005:1-2-1 |
| ONT Description | GI-NSOTESTEO-HGCL_TNOC_B07320NTC000056 |

### Bandwidth Profile Verification

| Parameter | Value |
| --- | --- |
| Upstream Bandwidth Profile | UP_TNET_GI_50M |
| Downstream Bandwidth Profile | DN_TNET_GI_50M |

### Network Device Verification

| Parameter | Value |
| --- | --- |
| OLT Host Name | tejas-ems-pkga |
| PE Host Name | NOCTESTMAR001 |
| PE Port | 0/4/0 |

### Response Verification

| Parameter | Value |
| --- | --- |
| Status Code | 2002 |
| Sub Status Code | 2002 |
| Message | Service Invoke Create Step2 Success |
| Transaction ID | 225 |
| Circuit ID | AC000E8930-01-04-000518-TL0005 |

Expected Result - Step 2 API is executed successfully. - Service-specific details are associated with Transaction ID 225. - ONT, OLT, PE Device, and bandwidth profile information are successfully processed by SCCM. - Circuit ID AC000E8930-01-04-000518-TL0005 is generated and returned in the response. - The transaction is updated and becomes available for Step 3 service provisioning.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Step 2 API Validation for GI-BB-TEJAS-SINGLE-ASR907-ROUTE Service Provisioning

## TC-10: Verify Step 3 API Execution and Service Provisioning

Service Order Reference: S109702026-09040205-022169

Description

Verify that the Step 3 Service Provisioning API is successfully executed for Service Order S109702026-09040205-022169 and that the service provisioning workflow is initiated using the Transaction ID generated during Step 1 and the service information submitted during Step 2.

Procedure

- Login to SCCM.

- Navigate to API Logs.

- Search for Transaction ID 225.

- Open the Step 3 API log entry.

- Verify the request payload details.

- Verify the response received from SCCM.

- Validate that the service provisioning request is successfully processed.

Validation Points

### API Details Verification

| Parameter | Value |
| --- | --- |
| Transaction ID | 225 |
| API URL | /sccm-dev/sccm_api/web/api/v1/servicedeploy/step3 |
| Request Method | POST |
| User | bss_test_user |

### Request Payload Verification

| Parameter | Value |
| --- | --- |
| ReqDtlID | 3390 |
| Transaction ID | 225 |
| Action | 1 |

### Response Verification

| Parameter | Value |
| --- | --- |
| Status Code | 2000 |
| Sub Status Code | 2000 |
| Message | Service Invoke Create Step3 Success |
| Unique ID | AC000E8930-01-04-000518-TL0005 |
| ReqDtlID | 3390 |

Expected Result - Step 3 API is executed successfully. - Transaction ID 225 is successfully processed. - Service provisioning workflow is initiated in SCCM. - Unique Service Identifier AC000E8930-01-04-000518-TL0005 is generated and returned in the response. - The service order is prepared for downstream RESTCONF-based orchestration towards NSO CFS. - A successful response message "Service Invoke Create Step3 Success" is returned by SCCM.

[ SCREENSHOT / IMAGE PLACEHOLDER ] Step 3 API Validation for GI-BB-TEJAS-SINGLE-ASR907-ROUTE Service Provisioning

## TC-11: Verify MPLS L3VPN RESTCONF Payload Generation and Transmission to NSO CFS

Service Order Reference: S109702026-09040205-022169

Description

Verify that after successful completion of Step 1, Step 2, and Step 3 service provisioning APIs, SCCM generates the MPLS L3VPN RESTCONF payload and sends it to NSO CFS for service orchestration. Validate that the payload contains the service, interface, VRF, QoS, and routing configuration details derived from the service order inputs.

Procedure

- Login to SCCM.

- Navigate to API Logs.

- Search for Transaction ID 225.

- Open the MPLS L3VPN RESTCONF API log.

- Verify the RESTCONF API URL and request method.

- Review the MPLS L3VPN payload.

- Validate the service, interface, VRF, QoS, and routing details.

- Verify that the payload is transmitted towards NSO CFS.

Validation Points

### RESTCONF API Verification

| Parameter | Value |
| --- | --- |
| Transaction ID | 225 |
| Request Method | PATCH |
| API URL | /restconf/data/tailf-ncs-services/mpls-l3vpn-cfs |

### Service Configuration Verification

| Parameter | Value |
| --- | --- |
| Service ID | S109702026-09040205-022169 |
| Customer Name | GI00000893 |
| Circuit ID | AC000E8930-01-04-000518-TL0005 |
| VPN ID | 20475 |
| Site ID | RFS-18 |
| Topology Name | topology1 |
| Routing Protocol | static |
| PE Type | single |
| Sub Service ID | 9.4.2.5 |

### Interface Configuration Verification

| Parameter | Value |
| --- | --- |
| Connected Device | NOCTESTMAR001 |
| Interface ID | Port-channel22 |
| Transaction ID | 225 |
| IP Type | IPv4 |
| Service Policy Name | Ingress-policy-GI |
| Customer VLAN | 3301 |
| Service Instance Number | 3301 |
| VRF Name | GI_BB_SERVICES_NSO |

### BDI Interface Verification

| Parameter | Value |
| --- | --- |
| Interface ID | BDI3301 |
| Connected Device | NOCTESTMAR001 |
| VRF Name | GI_BB_SERVICES_NSO |
| WAN IP Address | 10.10.15.1 255.255.255.240 |

### VRF Configuration Verification

| Parameter | Value |
| --- | --- |
| Device | NOCTESTMAR001 |
| VRF Name | GI_BB_SERVICES_NSO |
| Transaction ID | 225 |
| Route Distinguisher | 172.17.224.1:113 |
| Address Family | both |
| Import Route Target | 141688:32 |
| Export Route Target | 141688:31 |

### QoS Configuration Verification

| Parameter | Value |
| --- | --- |
| Device | NOCTESTMAR001 |
| QoS Name | COS_BRONZE |
| Transaction ID | 225 |
| Policy Map Name | Ingress-policy-GI |
| Class Map Name | GI_VLAN_3301 |
| QoS Group Value | 1 |
| Class Map VLAN | 3301 |

### Static Routing Verification

| Parameter | Value |
| --- | --- |
| Device | NOCTESTMAR001 |
| Transaction ID | 225 |
| BGP ID | 141688 |
| Provider AS Number | 141688 |
| IP Type | IPv4 |
| Redistribute Connected Required | Yes |
| VRF Name | GI_BB_SERVICES_NSO |
| Import Path Required | Yes |

Expected Result - MPLS L3VPN RESTCONF payload is successfully generated by SCCM. - The payload contains service, interface, VRF, QoS, and static routing configuration details. - The RESTCONF API is invoked using the PATCH method. - The payload is successfully transmitted to NSO CFS. - NSO CFS receives the MPLS L3VPN service configuration and initiates MPLS service orchestration. - The configuration data generated from SCCM provisioning inputs is available for subsequent validation within NSO CFS.

[ SCREENSHOT / IMAGE PLACEHOLDER ] MPLS L3VPN RESTCONF Payload Transmission from SCCM to NSO CFS

## TC-12: Verify MPLS L3VPN Service Creation in NSO CFS

Service Order Reference: S109702026-09040205-022169

Description

Verify that the MPLS L3VPN RESTCONF payload sent from SCCM is successfully received and processed by NSO CFS, and that the corresponding MPLS L3VPN service instance is created in the NSO CFS Service Manager.

Procedure

- Login to NSO CFS.

- Navigate to Services.

- Search for the Service ID: S109702026-09040205-022169.

- Open the service instance from the search results.

- Verify that the MPLS L3VPN service details page is displayed.

- Review the service information displayed in the Details tab.

Validation Points

### Service Details Verification

| Parameter | Value |
| --- | --- |
| Service ID | S109702026-09040205-022169 |
| Service Type | ncs:services/mpls-l3vpn-cfs:mpls-l3vpn-cfs |
| Modified Devices | 0 device(s) |
| Directly Modified Devices | 0 device(s) |
| Modified Services | 0 service(s) |
| Directly Modified Services | 0 service(s) |

### Additional Checks

- The service instance is accessible from the NSO CFS Services inventory.

- The service ID matches the value received through the SCCM RESTCONF payload.

- The MPLS L3VPN service instance is successfully created in NSO CFS.

Expected Result - NSO CFS successfully receives the MPLS L3VPN RESTCONF payload from SCCM. - A corresponding MPLS L3VPN service instance is created in NSO CFS. - The service instance is accessible using the Service ID S109702026-09040205-022169. - The service is displayed under the MPLS L3VPN service category in NSO CFS. - Successful creation of the service instance confirms that the RESTCONF payload transmitted from SCCM has been processed by NSO CFS.

[ SCREENSHOT / IMAGE PLACEHOLDER ] MPLS L3VPN Service Instance Creation Verification in NSO CFS

## TC-13: Verify MPLS L3VPN Service Creation in NSO RFS

Service Order Reference: S109702026-09040205-022169

Description

Verify that the MPLS L3VPN service orchestrated from NSO CFS is successfully created in NSO RFS and associated with the target network device and service components required for deployment.

Procedure

- Login to NSO RFS.

- Navigate to Services.

- Search for the Service ID: S109702026-09040205-022169.

- Open the service instance from the search results.

- Verify that the MPLS L3VPN service details page is displayed.

- Review the service information available in the Details tab.

Validation Points

### Service Details Verification

| Parameter | Value |
| --- | --- |
| Service ID | S109702026-09040205-022169 |
| Service Type | ncs:services/l3vpn:l3vpn |
| Modified Devices | 1 device(s) |
| Modified Services | 5 service(s) |
| Directly Modified Devices | 0 device(s) |
| Directly Modified Services | 5 service(s) |

### Additional Checks

- The service instance is available in NSO RFS.

- The service ID matches the MPLS L3VPN service received from NSO CFS.

- The modified device count and service count are populated.

Expected Result - NSO RFS successfully receives the MPLS L3VPN service from NSO CFS. - A corresponding MPLS L3VPN service instance is created in NSO RFS. - The service instance is accessible using Service ID S109702026-09040205-022169. - The service is associated with 1 target network device. - The service contains 5 service components created for deployment. - Successful creation of the RFS service confirms that the service orchestration request has progressed from NSO CFS to NSO RFS for device-level implementation.

[ SCREENSHOT / IMAGE PLACEHOLDER ] MPLS L3VPN Service Instance Verification in NSO RFS After CFS Orchestration

## TC-14: Verify ONT Creation and Status in Tejas EMS

Service Order Reference: S109702026-09040205-022169

Description

Verify that the ONT provisioned through SCCM and orchestrated via NSO is successfully created in Tejas EMS. Validate that the ONT is associated with the correct OLT, serial number, equipment type, and operational status.

Procedure

- Login to Tejas EMS.

- Navigate to Manage ONTs.

- Search using the ONT Serial Number: TJNW95001D3F.

- Open the ONT entry displayed in the search results.

- Verify the ONT details and status information.

Validation Points

### ONT Details Verification

| Parameter | Value |
| --- | --- |
| Node Name | NOCTESTOLT005 |
| Product Type | TJ1400_Type-13SR |
| OLT Status | AVAILABLE |
| ONT Name | ONT-1-2-11-1 |
| Equipment ID | TJ210N-11G |
| EMS State | ACTIVATE |
| Physical Status | Activated |
| Admin State | Enable |
| Serial Number | TJNW95001D3F |
| Location | TNOC |
| Distance (km) | 0.0 |

### Additional Checks

- The ONT Serial Number matches the value provisioned through SCCM.

- The ONT is associated with the expected OLT node.

- The EMS State is displayed as ACTIVATE.

- The Physical Status is displayed as Activated.

- The Administrative State is displayed as Enable.

- The ONT is available for service provisioning on the target OLT.

Expected Result - ONT is successfully created in Tejas EMS. - ONT details match the service provisioning information sent from SCCM through NSO. - ONT is associated with the correct OLT device. - EMS and Physical Status indicate that the ONT is active. - The successful ONT creation confirms that the GPON service configuration has been deployed on the Tejas access network infrastructure.

[ SCREENSHOT / IMAGE PLACEHOLDER ] ONT Creation and Status Verification in Tejas EMS After Successful NSO Orchestration

# 3. Test Summary and Sign-off

This section records the overall test outcome and formal sign-off for the acceptance test execution of both service orders: S109702026-09050204-022073 (Nokia) and S109702026-09040205-022169 (Tejas).

| Device | Test Case | Title | Result (Pass / Fail) |
| --- | --- | --- | --- |
| Nokia | TC-01 | Step 1 – Service Order Initialization |  |
| Nokia | TC-02 | Step 2 – Service Order Data Submission |  |
| Nokia | TC-03 | Step 3 – Service Provisioning |  |
| Nokia | TC-04 | MPLS L3VPN RESTCONF Payload to NSO CFS |  |
| Nokia | TC-05 | GPON RESTCONF Payload to NSO CFS |  |
| Nokia | TC-06 | ONT Status, ONT Details & VLAN Verification |  |
| Nokia | TC-07 | Internet Bandwidth Validation |  |
| Tejas | TC-08 | Service Order Creation and Step 1 API Execution |  |
| Tejas | TC-09 | Step 2 API Execution and Service Data Submission |  |
| Tejas | TC-10 | Step 3 API Execution and Service Provisioning |  |
| Tejas | TC-11 | MPLS L3VPN RESTCONF Payload to NSO CFS |  |
| Tejas | TC-12 | MPLS L3VPN Service Creation in NSO CFS |  |
| Tejas | TC-13 | MPLS L3VPN Service Creation in NSO RFS |  |
| Tejas | TC-14 | ONT Creation and Status in Tejas EMS |  |

| Parameter | Value |
| --- | --- |
| Tested By |  |
| Designation |  |
| Date |  |
| Signature |  |
| Approved By |  |
| Designation |  |
| Date |  |
| Signature |  |